Assignment 4

Configuration of DHCP Server on Router using Cisco Packet Tracer

## Given Requirements

- DHCP pool name: xyz

- Network: 192.168.10.0/24

- DNS Server: 1.2.3.4

- Lease time: 3 hours

- Router should not respond to BOOTP

- Clients should obtain IP automatically

## Topology Description

A router is connected to a switch, and two PCs are connected to the switch.
The router acts as a DHCP server, assigning IP addresses to the PCs.

## Connections

- Router → Switch (Straight cable)

- PC1 → Switch (Straight cable)

- PC2 → Switch (Straight cable)

## IP Addressing Plan

| Device | IP Address |
| --- | --- |
| Router | 192.168.10.1 |
| PC1 | DHCP |
| PC2 | DHCP |

## Configuration Steps

### Step 1: Configure Router Interface

enable
conf t
interface g0/0
ip address 192.168.10.1 255.255.255.0
no shutdown
exit

### Step 2: Configure DHCP Server

ip dhcp excluded-address 192.168.10.1

ip dhcp pool xyz
network 192.168.10.0 255.255.255.0
default-router 192.168.10.1
dns-server 1.2.3.4

👉 (Note: Lease command may not work in Packet Tracer, but theoretically it is 3 hours)

### Step 3: Configure Clients

- Go to PC → Desktop → IP Configuration

- Select DHCP

## Verification

### Check IP on PC

ipconfig

### Test Connectivity

ping 192.168.10.1

### Check DHCP Binding on Router

show ip dhcp binding

Topology

Commands to setup dhcp

Automatically assigning the ip address using dhcp.

Checking dhcp binding and dhcp pool

Pinging from one pc to another.