Assignment 5

Ques 1:

# STEP 0: What you are building

👉 2 networks:

- Left: 192.168.1.0/24

- Right: 172.16.0.0/16

- Connected using 2 routers via serial cable

STEP 1: Add Components

## 🎯 What to drag

From bottom panel:

- 2 Routers → (2811)

- 2 Switches → (2960)

- 2 PCs

👉 What you will see:

- Devices appear in workspace

- Names like: Router0, Router1, PC0, PC1

# STEP 2: Connections (VERY IMPORTANT)

## 1️⃣ PC1 → Switch1

- Click ⚡ (Connections)

- Select Copper Straight

- Click:

  - PC1 → FastEthernet0

  - Switch1 → Fa0/1

👉 What you see:

- Cable appears

- Orange → Green light (after few seconds)

2️⃣ Switch1 → Router1

- Straight cable

- Switch1 Fa0/2 → Router1 Fa0/0

## 3️⃣ PC2 → Switch2

- Straight cable

- PC2 Fa0 → Switch2 Fa0/1

## 4️⃣ Switch2 → Router2

- Straight cable

- Switch2 Fa0/2 → Router2 Fa0/0

## 5️⃣ Router1 ↔ Router2 (MOST IMPORTANT)

- Select Serial DCE cable

- Click:

  - Router1 → Serial1/0

  - Router2 → Serial1/0

👉 What you see:

- One side shows clock icon ⏰ (DCE)

# STEP 3: Configure Router R1

👉 Click Router1 → CLI

👉 You will see:

Would you like to enter initial configuration dialog? [yes/no]

👉 Type:

no

### Enter commands:

enable
conf t
interface fa0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
exit

👉 What you see:

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

### Serial config:

interface s1/0
ip address 10.0.0.1 255.255.255.252
clock rate 64000
no shutdown
exit

👉 What you see:

- Interface becomes up

# STEP 4: Configure Router R2

👉 Same process:

enable
conf t
interface s1/0
ip address 10.0.0.2 255.255.255.252
no shutdown
exit

interface fa0/0
ip address 172.16.0.1 255.255.0.0
no shutdown
exit

# STEP 5: Verify Interfaces

show ip interface brief

👉 What you see:

Fa0/0    up    up
Serial1/0 up   up

✔ If not → check cables or no shutdown

# STEP 6: Check Routing Table (Before)

show ip route

👉 You will see:

- Only directly connected networks

# STEP 7: Configure Static Routing

## On R1:

ip route 172.16.0.0 255.255.0.0 10.0.0.2

👉 Meaning:

- To reach 172 network → go via R2

## On R2:

ip route 192.168.1.0 255.255.255.0 10.0.0.1

# STEP 8: Verify Static Route

show ip route

👉 What you see:

S 172.16.0.0 [1/0] via 10.0.0.2

✔ S = Static route

# STEP 9: Configure PCs

## PC1:

- Desktop → IP Configuration

Fill:

- IP: 192.168.1.100

- Mask: 255.255.255.0

- Gateway: 192.168.1.1

## PC2:

- IP: 172.16.0.100

- Mask: 255.255.0.0

- Gateway: 172.16.0.1

👉 What you see:

- No error

- IP shown in fields

# STEP 10: TEST (MOST IMPORTANT)

## From PC1:

ping 172.16.0.100

👉 First few:

Request timed out

👉 Then:

Reply from 172.16.0.100

✔ Means working

Q.1 Objective: Configure Static Route in Cisco packet tracer

Topology

Router configurations

Pinging pc1 to pc2

Q2

Router 1 Commands

Router 2 Commands

Ping from Pc 1 to Pc 2

Question 3:

Router Commands

Ping from 1st network’s PC

Ping from 2nd network’s PC