ASSGNMENT 6: To configure and verify routing using RIP Version 2 and version 1  protocol in a network.

# 🟦 🔰 STEP 0: What you are building

👉 3 Routers:

- R1 (center)

- R2 (left LAN)

- R3 (right LAN)

👉 Networks:

- R1 ↔ R2 → 100.1.1.0

- R1 ↔ R3 → 20.1.1.0

- Left LAN → 172.16.2.0

- Right LAN → 10.2.2.0

👉 Goal:
✔ Use RIP v2 routing
✔ All networks should communicate

# 🟦 🔌 STEP 1: Add Components

👉 Drag:

- 3 Routers (2811 / 2911)

- 2 Switches (2960)

- 2 PCs

👉 What you see:

- Router0, Router1, Router2

- PC0, PC1

# 🟦 🔌 STEP 2: Connections (VERY IMPORTANT)

## 🔹 Left side

- PC1 → Switch1 (Straight)

- Switch1 → Router2 (g0/0)

## 🔹 Right side

- PC2 → Switch2 (Straight)

- Switch2 → Router3 (g0/0)

## 🔹 Router connections

- R1 ↔ R2 → Serial DCE

- R1 ↔ R3 → Serial DCE

👉 What you see:

- Red → turns Green after config

- One side shows ⏰ (DCE)

# 🟦 ⚙️ STEP 3: Configure Router R1

👉 CLI:

enable
conf t

## Interface 1:

interface s0/0/0
ip address 100.1.1.2 255.255.255.0
clock rate 64000
no shutdown

👉 What you see:

%LINK-5-CHANGED

## Interface 2:

interface s0/0/1
ip address 20.1.1.1 255.255.255.0
no shutdown

## Enable RIP:

router rip
version 2
no auto-summary
network 100.1.1.0
network 20.1.1.0

👉 What happens:
✔ R1 starts sharing routes

# 🟦 ⚙️ STEP 4: Configure Router R2

enable
conf t

## Serial:

interface s0/0/0
ip address 100.1.1.1 255.255.255.0
no shutdown

## LAN:

interface g0/0
ip address 172.16.2.1 255.255.0.0
no shutdown

## RIP:

router rip
version 2
no auto-summary
network 100.1.1.0
network 172.16.0.0

# 🟦 ⚙️ STEP 5: Configure Router R3

enable
conf t

## Serial:

interface s0/0/1
ip address 20.1.1.2 255.255.255.0
no shutdown

## LAN:

interface g0/0
ip address 10.2.2.1 255.255.255.0
no shutdown

## RIP:

router rip
version 2
no auto-summary
network 20.1.1.0
network 10.2.2.0

# 🟦 💻 STEP 6: Configure PCs

## PC1:

- IP: 172.16.2.10

- Mask: 255.255.0.0

- Gateway: 172.16.2.1

## PC2:

- IP: 10.2.2.20

- Mask: 255.255.255.0

- Gateway: 10.2.2.1

# 🟦 🔍 STEP 7: Verify Interfaces

show ip interface brief

👉 You should see:

up   up

# 🟦 🔍 STEP 8: Verify RIP Routing

show ip route

👉 You will see:

R 172.16.0.0
R 10.2.2.0

✔ R = RIP learned route

# 🟦 📡 STEP 9: Test Connectivity

## From PC1:

ping 10.2.2.20

👉 First few:

Request timed out

👉 Then:

Reply from 10.2.2.20

✔ Success

Topology

Comands

Pinging