Assignment 7

Routing using OSPF (Link State Routing) in Cisco Packet Tracer

# 🧩 STEP 1: DEVICES REQUIRED

- 🟦 3 Routers → R1, R2, R3

- 🟩 2 Switches → SW1, SW2

- 💻 2 PCs → PC1, PC2

# 🔌 STEP 2: CONNECTIONS (VERY IMPORTANT)

## ✅ 1. PC ↔ Switch

- Cable: Copper Straight-Through

- Ports:

PC → FastEthernet0
Switch → FastEthernet0/1

## ✅ 2. Switch ↔ Router

- Cable: Copper Straight-Through

- Ports:

SW1 → R2 G0/0
SW2 → R3 G0/0

## ✅ 3. Router ↔ Router (Serial Links)

### 🔌 Cable:

👉 Serial DCE

### Connections:

R1 S0/1/0 ↔ R2 S0/2/0
R1 S0/1/1 ↔ R3 S0/2/0

⚠️ One side must have:

clock rate 64000

# 🌐 STEP 3: IP ADDRESSING (AS PER DIAGRAM)

## 🔹 R1

S0/1/0 → 100.1.1.1 /24
S0/1/1 → 20.1.1.1 /24

## 🔹 R2

S0/2/0 → 100.1.1.2 /24
G0/0 → 172.16.2.1 /24

## 🔹 R3

S0/2/0 → 20.1.1.2 /24
G0/0 → 10.2.2.1 /24

## 💻 PCs

### PC1 (Left LAN)

IP: 172.16.2.10
Mask: 255.255.255.0
Gateway: 172.16.2.1

### PC2 (Right LAN)

IP: 10.2.2.20
Mask: 255.255.255.0
Gateway: 10.2.2.1

# ⚙️ STEP 4: ROUTER CONFIGURATION

## 🟦 R1 CONFIG

enable
conf t

int s0/1/0
ip add 100.1.1.1 255.255.255.0
clock rate 64000
no shut

int s0/1/1
ip add 20.1.1.1 255.255.255.0
no shut

## 🟦 R2 CONFIG

enable
conf t

int s0/2/0
ip add 100.1.1.2 255.255.255.0
no shut

int g0/0
ip add 172.16.2.1 255.255.255.0
no shut

## 🟦 R3 CONFIG

enable
conf t

int s0/2/0
ip add 20.1.1.2 255.255.255.0
no shut

int g0/0
ip add 10.2.2.1 255.255.255.0
no shut

# 🔁 STEP 5: OSPF CONFIGURATION

From your PDF (page 2)

## 🔹 R1

router ospf 1
router-id 1.1.1.1
network 100.1.1.0 0.0.0.255 area 0
network 20.1.1.0 0.0.0.255 area 0

## 🔹 R2

router ospf 1
router-id 2.2.2.2
network 100.1.1.0 0.0.0.255 area 0
network 172.16.2.0 0.0.0.255 area 0

## 🔹 R3

router ospf 1
router-id 3.3.3.3
network 20.1.1.0 0.0.0.255 area 0
network 10.2.2.0 0.0.0.255 area 0

# ✅ STEP 6: VERIFICATION

## 🔍 Check neighbors

show ip ospf neighbor

## 🔍 Check routing table

show ip route

You should see:

O 172.16.2.0
O 10.2.2.0

## 🔍 Ping test

From PC1:

ping 10.2.2.20

✔ Should work ✅