# Assignment 8

# Border Gateway Protocol

# 🧩 STEP 1: DEVICES REQUIRED

- 🟦 3 Routers → R1, R2, R3

- 🟩 3 Switches (optional but recommended)

- 💻 3 PCs (one per LAN)

# 🔌 STEP 2: CONNECTIONS (VERY IMPORTANT)

## ✅ 1. PC ↔ Switch

- Cable: Copper Straight-Through

PC → FastEthernet0
Switch → FastEthernet0/1

## ✅ 2. Switch ↔ Router

- Cable: Copper Straight-Through

Switch → Router G0/0

## ✅ 3. Router ↔ Router

👉 Use Copper Cross-Over OR Straight (auto works in PT)

R1 G0/1 ↔ R2 G0/0
R2 G0/1 ↔ R3 G0/0

# 🌐 STEP 3: IP ADDRESSING (FROM PDF)

## 🟦 R1

G0/0 → 192.168.1.1 /24
G0/1 → 10.0.0.1 /30

## 🟦 R2

G0/0 → 10.0.0.2 /30
G0/1 → 10.0.0.5 /30

## 🟦 R3

G0/0 → 10.0.0.6 /30
G0/1 → 192.168.3.1 /24

# 💻 PC CONFIGURATION

## PC1 (R1 LAN)

IP: 192.168.1.2
Mask: 255.255.255.0
Gateway: 192.168.1.1

## PC2 (R2 LAN – optional)

IP: 10.0.0.10
Mask: 255.255.255.252
Gateway: 10.0.0.2

## PC3 (R3 LAN)

IP: 192.168.3.2
Mask: 255.255.255.0
Gateway: 192.168.3.1

# ⚙️ STEP 4: ROUTER CONFIGURATION

## 🟦 R1 CONFIG (AS 100)

enable
conf t

int g0/0
ip add 192.168.1.1 255.255.255.0
no shut

int g0/1
ip add 10.0.0.1 255.255.255.252
no shut

## 🟦 R2 CONFIG (AS 200)

enable
conf t

int g0/0
ip add 10.0.0.2 255.255.255.252
no shut

int g0/1
ip add 10.0.0.5 255.255.255.252
no shut

## 🟦 R3 CONFIG (AS 300)

enable
conf t

int g0/0
ip add 10.0.0.6 255.255.255.252
no shut

int g0/1
ip add 192.168.3.1 255.255.255.0
no shut

# 🌍 STEP 5: BGP CONFIGURATION

(From your PDF page 3)

## 🔹 R1 (AS 100)

router bgp 100
neighbor 10.0.0.2 remote-as 200
network 192.168.1.0 mask 255.255.255.0

## 🔹 R2 (AS 200)

router bgp 200
neighbor 10.0.0.1 remote-as 100
neighbor 10.0.0.6 remote-as 300
network 10.0.0.0 mask 255.255.255.252

## 🔹 R3 (AS 300)

router bgp 300
neighbor 10.0.0.5 remote-as 200
network 192.168.3.0 mask 255.255.255.0

# ✅ STEP 6: VERIFICATION

## 🔍 Check BGP neighbors

show ip bgp summary

✔ Should show Established

## 🔍 Check routes

show ip route

✔ Look for:

B 192.168.3.0
B 192.168.1.0

## 🔍 Ping test

From PC1:

ping 192.168.3.2

✔ Should work ✅

Topology

Commands